Project Manager:  Audrey Van Cleve

Hydrologist:  Audrey Van Cleve

Consultant:  Liesch, Aaron Benker, 763 489-3147

7/16/12 AVC review of FI Workplan received 6/22/12:

LNAPL checks since last report performed on 3/29, 4/27, and 5/30/12.

MW3 had 3.5 inches product in May, none in March or April.

MW7 had no product.  MW7 had 16 inches in Jan 2010 and 1.8 inches in Jan and Mar 2011.  No product from 2005 until Jan 2011.  Product was present during times of highest water elevations.  It should be noted that the sampling schedule has been very inconsistent with no sapling conducted from July 2007 until Jan 2010, then nothing until Jan 2011.

Proposed FI:  Prior to conducting LIF, advance one probe at the location of a proposed LIF/EC probe for calibrating EC.  Screen with a PID; no lab samples to be collected.

Propose 7 LIF/EC probes as 1st tier:  two north of LIF 19, two north of on-site building, and three south of 2nd Ave.  Site map shows locations of 2nd tier probes that will be advanced as necessary to achieve readings >2% for lateral definition.  Probes to be advanced to 10 ft below deepest measureable LNAP or 10 ft below the water table, whichever is deeper.  Anticipated depths are 30 ft.  Site map with proposed LIF locations shows a grid of 40 to 50 ft on the north property boundary and 75 to 100 ft south of 2nd Ave.  Grid needs to be tightened up in accordance with GD 7-01 Appendix B LIF/EC Guidance which specifies a 25- to 35-foot grid.

Install MW8 in the TH15 location, MW9 in the TH16 location, MW10 on south property boundary to provide triangular arrangement for better GW flow direction evaluation, and MW11 midway between MW6 and Kluck Oil (225 ft SW of MWs 6) for downgradient dissolved phase delineation.

Recommendations:

Approve the Focused Investigation Work Plan signed on 6/20/2012 with the following modification:  The grid spacing between LIF/EC probes must be tightened in accordance with MPCA GD 7-01 Appendix B LIF/EC Guidance which specifies a 25- to 35-foot grid.