Correction Review Documentation  						Date:  9/29/2015

07010203-510 Mississippi River; Clearwater R to Elk R

Pollutant or stressor:  Impaired Biota - Fish

First listed:    2002

Original dataset:

| List Year | Station_ID | Results | Parameter Name | Date | Assessment | Chronic Std |
| --- | --- | --- | --- | --- | --- | --- |
| 2002 | 00UM092 | 54 | IBI Fish | 8/29/2000 | NS | 61 |
| 2002 | 00UM092 | 62 | IBI Fish | 9/14/2000 | FS | 61 |
| 2002 | 99UM034 | 76 | IBI Fish | 8/17/1999 | FS | 61 |

Reason for Listing / Additional information:  Previous threshold for non-support utilizing outdated biological criteria from Upper Mississippi River Basin IBI was an F-IBI score of 61 or below. The AUID was originally listed during the 2002 Reporting Cycle because data from one of three visits (2 stations) indicated non-support for aquatic life based on an F-IBI score of 54.

Delisting/Correction requirements:

Guidance Manual for Assessing the Quality of Minnesota Surface Waters for Determination of Impairment: 305(b) Report and 303(d) List, 2014 Assessment and Listing Cycle

D. List correction (p. 41)

If a waterbody was placed on the list in error either by a wrong AUID being assigned to the data or due to an update in a standard or methodology that would not have caused an initial listing, the reach will be removed from the list as a correction.

Current assessment (2015):

Original Listing data re-scored with new criteria:

| List Year | Station_ID | Results | Parameter Name | Date | Assessment | Chronic Std |
| --- | --- | --- | --- | --- | --- | --- |
| 2016 | 00UM092 | 49.5 | IBI Fish | 8/29/2000 | MTS | 38 + 9 |
| 2016 | 00UM092 | 42.7 | IBI Fish | 9/14/2000 | MTS | 38 + 9 |
| 2016 | 99UM034 | 53.8 | IBI Fish | 8/17/1999 | MTS | 38 + 9 |

Biological Review:  M. Feist - 9/29/15 – A correction to the impaired waters list is being proposed for this AUID based on the development of new statewide biological criteria since the initial listing. When older listing data are evaluated with new criteria and the resulting assessment indicates support, then the AUID should be considered for removal from the impaired waters list as a correction.

The original listing indicated non-support for aquatic life based on biological criteria and assessment methodology in place at the time. Utilizing new statewide biological indices and criteria on the original dataset now results in F-IBI scores that are all above the threshold (two visits above threshold and upper confidence interval and one visit above threshold within CI) and indicate that aquatic life use is supporting based on fish assessment. A recommendation of support for aquatic life (F-IBI) and a correction to the 303d list is proposed.

It should be noted that this AUID (07010203-510) is now part of consolidated large river assessment units (based on HUC10 linework) and is now within AUID 07010203-729. For 305b/303d purposes the parent AUID’s are still being maintained and the correction should occur to AUID 510 for reporting and tracking.

Watershed Assessment Team:

Additional information:

Correction Review meeting summary & (date):

Recommendation:  Correction to impaired waters list based on new statewide biological criteria.