Request Form for Assessing Lakes for Natural Background Conditions

In order to make an informed decision, the lake/stream water chemist must have as much information as possible about the lake in question in order to determine if natural background conditions are responsible for eutrophication.  Champions should gather as much of the following information as possible and provide it to the lake/stream water chemist (typically a Lakes & Streams Monitoring Unit staff) for the watershed in question.  This form, with any supporting documentation attached, must be delivered one week prior to the scheduled professional judgment group (PJG) meeting. Depending on how much time is available during the PJG, a follow-up meeting to the PJG may be needed to finish discussion and reach a decision.

| Champion’s name: |  |  |  |
| --- | --- | --- | --- |
| Cover | Recommended Sources: | Lake name (ID#): Lows 18018000 | Ecoregion Specific Land Use Ranges from table below: |
| Lake Ecoregion | US EPA Level III (revised 2000) | Northern Lakes and Forest |  |
| Lake surface area | 1:24,000 NDH area | 319 acres |  |
| Lake maximum depth | DNR Fisheries | 2.7 m |  |
| Lake mean depth |  | 1.6 m |  |
| Lake % littoral |  | 100% |  |
| Watershed area (total) | DNR Catchment layer | 5827 acres |  |
| Watershed: lake area |  | 18:1 |  |
| Forest (% total) | 2001 NLCD Land Use (or newer) | 63% | 54 – 81 |
| Water/wetland (% total) |  | 35% | 14 – 31 |
| Pasture/open (% total) |  | <1% | 0 – 6 |
| Cultivated (% total) |  | 0% | < 1 |
| Urban (% total) |  | 1.5% | 0 – 7 |
| Assessed data (# obs) | MPCA STORET data | 8(2012, 2013) |  |
| TP µg/L (10-year average) |  | 65 ug/l |  |
| Chl-a µg/L (10-year average) |  | 36.5 ug/l |  |
| Secchi meters (10-year average) |  | 0.9 m |  |
| IWM year | MPCA | 2012 |  |
| Local sampling (years) |  | 2 |  |
| Secchi trend (from CLMP trends) | MPCA | Insufficient Data |  |
| Watershed name (HUC#) |  | 07010105 |  |

Champion can ask assigned lake chemist (usually someone from the Lakes & Streams Monitoring Unit) for help in completing the table above.

| Land Use % | NLF (NMW) | NCHF (DA & RRV) | WCBP | NGP |
| --- | --- | --- | --- | --- |
| Forest | 54 – 81 | 6 – 25 | 0 – 15 | 0 – 1 |
| Water and Wetland | 14 – 31 | 14 – 30 | 3 – 26 | 8 – 26 |
| Pasture and Open | 0 – 6 | 11 – 25 | 0 – 7 | 5 – 15 |
| Cultivated | < 1 | 22 – 50 | 42 – 75 | 60 – 82 |
| Urban | 0 – 7 | 2 – 9 | 0 – 16 | 0 - 2 |

Notes:

- If you believe that the lake in question should be assessed against alternative ecoregion standards, that request should be taken directly to the watershed assessment team during the professional judgment group meeting.  Such cases will not be considered through a natural background review process.

- Direct point source discharges to the lake or notable point/nonpoint source activity within the lake’s watershed will likely result in a negative decision for the natural background request.  See the guidance document for more information.

- Have there been any changes to fisheries management in recent years?  Fisheries staff at the nearest DNR regional and area offices can help staff determine this.

- Lows is not currently or historically managed for any specific fish species. A survey conducted in 1984 shows naturally reproducing populations of northern pike, black crappie, bluegill, largemouth bass, yellow perch and bullhead are common in Lows.

- Provide a summary, and the locations on a map, of ditching or stormwater conveyances that lead directly to the lake.  Local (i.e., county) records may be available to help staff determine this.

- There are no channelized waterways or stormwater conveyances that input into this lake basin. The natural channel of Mud Brook inflows into the Lows Southeast corner and outflows in the Northwest corner.

- Provide information on the number of cabins, year-round lake homes, and resorts on the lake shoreline, and a map showing the location of this development.

- Using county online GIS mapping there is 12 parcels with dwellings on the shoreline and appear to be a mix of seasonal cabins and year round residence.

- Provide summary information as to the status of septic systems of dwellings on the shoreline. Local offices may have such records.

- Using Crow Wing county’s online interactive GIS mapping and permit search there are 5 parcels with dwellings on the lake shore that have permitted septic systems that were installed from 1996-2012. Compliance records are not readily available; it appears only one newly installed SSTS in 2012 can be confirmed in compliance from online records. The county office was contacted and was unable to provide any more information than what is available online.

- Provide information pertaining to changes to aquatic vegetation in the lake over time.  Has there been chemical or mechanical removal of vegetation in response to shoreland development?  Staff may be able to consider changes over time through aerial photos or by contacting the aquatic plant management (APM) staff in the DNR fisheries office nearest the lake to see if APM permits have been issued in the past.

- A wildlife habitat survey done by DNR shallow lake program notes vegetation in a natural state throughout the lake and that the center of the lake was generally devoid of aquatic plant growth. Lows is managed as a wild rice lake by Brainerd DNR wildlife staff, producing about 50 acres of wild rice habitat on a good year.

- Are their permitted point source discharges (WWTFs or feedlots) to the lake in question or within the watershed?  If yes, please provide details on the nature of the discharge, the maximum capacity (MGD for WWTFs, maximum animal units for feedlots) the receiving water, proximity to the lake, etc.

- No permitted WWTFs or feedlots occurred within this small watershed

- Provide any known information on unpermitted activities that occur on or near the lakeshore (i.e., unpermitted feedlot activities, etc.).

- Using aerial photography there does not appear to be any small unpermitted feedlot activity on the immediately lakeshore. Reviewing NLCD 2011 and aerials there does appear to be a small pasture 0.4 miles NE of the SE shore of the lake that is included in the Lows lake watershed.

- Provide information pertaining to the historical use of, or impacts to, the lake (i.e., previous feedlot operations or point-source discharge, previous development around the lake, etc.).  Local staff and even landowners can often provide this perspective. An approach for assembling/researching historical land use and related activities from county/local records and archives is presented in Heiskary and Swain (2002).

- Limited local input was offered on Lows Lake during the Pine River Professional Judgment Group meeting. There is a silo and older out building foundations on the lakeshore of Lows in the NW corner which would suggest historical farming activity, reviewing historical aerial photography from 1955 and 1938 confirms a farm was once operating in this location.

- Have lake sediment cores been collected from the lake (or a nearby lake) that were used to reconstruct pre-European phosphorus concentrations or other relevant information? (materials that can help with this are located at: X:\Agency_Files\Water\Condition Monitoring\Lakes\Sediment diatom work

- Lows has not received any sediment core work. The nearest lakes are roughly 50 miles to the north(Winnibigoshish) and south(Red Sand). Neither lake seems to share significant characteristics in common with Lows.

Appendix C. Natural Background Evaluation form

Evaluation Form for Determination of Nutrient Impairment in Lakes Due to Natural Background Conditions

GIS project is called NatBackground.mxd and is located in X:\Agency_Files\Water\303D List\Natural Background Folder\Lakes.

| Review date | 4/22/15 |  |  |  |
| --- | --- | --- | --- | --- |
| Lake name; ID # | Lows 18018000 |  |  |  |
| Review team members | Jordan Donatell, Dave Christopherson, Chandra Carter, Lee Engel, Carol Sinden, Scott Niemela, Dave Dollinger, Ben Lundeen, Chuck Johnson |  |  |  |
| Stressor |  | Present in watershed (yes/no) | Stressor contribution potential (high, moderate, low, none) | Comments (see below for possible factors to consider in evaluating each stressor) |  |
| Anthropogenic sources |  |  |  |  |  |
| Overall land use disturbance |  | yes | low | 12 dwellings on shoreline, 5 permitted septics, one road adjacent to lake, some forest clearing for hay production, small pasture, and power lines, no ditching or stormwater conveyances, minimal human development |  |
| Forest harvest sites |  | Yes | Low | Forest Activity is present in the northern and southern edge of the watershed, this appears to a mix of selective and clear cutting practices |  |
| Row crop agriculture |  | no |  |  |  |
| Pasture and hay land |  | yes | low | Small area of clearing near the north end of the lake, another cleared area south of the lake which appears to be hayfield, cleared pasture/hay field on the northern edge of the watershed, forest clearing for large power lines also is evident |  |
| Roads |  | yes | low | County Road 1 in the north side of the watershed and lake |  |
| Feedlots (permitted) |  | No |  |  |  |
| Feedlots (unpermitted) |  | No |  |  |  |
| Direct conveyances (ditches or streamflows) to lake |  | Yes | Low | Mud brook, another small tributary |  |
| Shoreline development (cabins, lake homes, resorts) |  | yes | low | 12 dwellings on the shoreline, 5 permitted septic’s only 1 can be deemed compliant due to incomplete records |  |
| Unsewered communities |  | No |  |  |  |
| Industrial or municipal wastewater facilities |  | No |  |  |  |
| Stormwater |  | No |  |  |  |
| Stormwater unpermitted |  | No |  |  |  |
| Removal of aquatic vegetation |  | No |  |  |  |
| Reservoirs and dams (manmade) |  | No |  |  |  |
| History of anthropogenic use of lake or within watershed |  | Yes | low | Historical aerial photography from 1938 and 1955 indicates a farming operation existed on the Northwest shore of Lows. |  |
| Do not proceed if anthropogenic factors suggest nutrient loading is measurable. Lake is not a candidate for natural background |  |  |  |  |  |
| Natural Factors |  |  |  |  |  |
| Forest cover |  | yes |  | ~63% of this small watershed is forested, forest activity is visible using aerial photography and related forest layers |  |
| Wetland cover |  | yes |  | ~35% of this small watershed, wetlands are abundant, Riparian wetlands are present along Mud Brook and Lows shoreline and could potentially contribute vegetation/organic material that naturally reduces transparency and are sources of nutrients to the lake |  |

Natural background review team recommendation. Include CALM designation or follow-up monitoring recommendation. Combine and attach all natural background evaluation forms used to evaluate an AUID.

12/28/2015: The natural background committee agreed with the recommendation to move Lows Lake to category 4D, impaired due to natural causes. There are 12 properties on the lake, 5 with permitted septic systems. The lake is very shallow and has a relatively large, wetland dominated watershed. If all systems were complaint, septics would make up an estimated 1% of the load to the lake, based on Reckhow Simpson modeling. If all were noncompliant, the estimated load is 2%. The group came to an agreement that this contribution is insignificant (unlikely to be measurable by monitoring).

One evaluation form is required for each lake. The evaluation of stressor potential is based primarily on the proximity of the stressor within the watershed and the degree of disturbance relative to the size of the waterbody. All stressors identified have the potential to contribute to a nutrient impairment through one or more pathways.

In the evaluation of all stressors consider the following evaluation criteria (USEPA 2008):

- Does the source contain or emit any of the critical parameters?

- Are there one or more pathways?

- Is the key parameter persistent enough to impact the reach?

- Does the source discharge the key parameters in the same time period that the impairment occurs?

The determination of natural condition will be based upon a weight of evidence approach utilizing the stressors and criteria defined below:

Process:

- Determine and view catchment and watershed boundaries;

- Turn on aerial photos and view watershed and lake;

- View stream flow lines and pourpoint of lake to determine if there are conveyances into the lakes and how water drains from the lake;

- Systematically turn on and off layers in GIS tool to consider the natural and anthropogenic impacts potentially affecting the lake;

- Consider additional information brought forward by champion (i.e., past historical use of lake, past anthropogenic activity in watershed, changes to fisheries management, presence of ditches or streams to lake that cannot be seen on GIS layers, nature of septic systems along the lakeshore, presence of shoreline disturbance that can’t be viewed by GIS layers, etc.);

- Consider assessment information and data; and

- Record findings and natural background team decision on form above.

Factors to consider in evaluating the potential contribution from each stressor:

Industrial or Municipal Wastewater Facilities

- Location/proximity of discharge to lake or within lake catchment

- Type of wastewater (noncontact cooling water, POTW, industrial discharge)

- Discharge pollutant levels vs. receiving water pollutant levels

- Discharge volume vs. receiving water volume

- Discharge type (seasonal, daily, land application)

Feedlots (permitted)

- Location/proximity to lake, catchment or upstream conveyance to lake

- Verify accuracy of GIS points using aerial photos

- Permit compliance (no discharge); fish kill history?

- Manure application areas in relation to lake, catchment or upstream conveyance to lake - on tile fields within lake catchment?  Cattle in lake?

Stormwater (permitted)

- Location/proximity to lake and shoreland

- Discharge volume vs. receiving water volume

- Type of stormwater (likely stressors include fertilizer/nutrients and animal waste)

- What stormwater controls are in place?

Reservoirs and dams (man-made)

- Location/proximity to lake and shoreland

- Nutrient status of the reservoir, if known

- Type of release (surface vs. hypolimnetic)

- Water level manipulation, eutrophic ambient conditions

Feedlots/pasture (unpermitted)

- Location/proximity to lake and shoreland

- Fish kill history

- Manure application areas in relation to lake, catchment or upstream conveyance to lake, cattle in lake, manure application on tiled fields within lake catchment

Forestry/row crop/pasture and hay lands

- Location/proximity to lake and shoreland

- Percentage of watershed in row crop

- Type of terrain (hilly, soil type, etc)

Physical alterations to the lake or shoreland

- Is the shoreland intact around the lake?

- Is there development within the lake’s catchment?

- Dams and reservoirs present within the watershed?

- Are channelized streams present within the watershed?

- Likelihood of agricultural tile drainage within system

Natural factors

- Is the lake’s catchment undisturbed by human-induced land use conversion?

- Is the lake a flow-through lake?

  - If so, is there any human disturbance of upstream lakes’ catchment?

  - Is/are upstream lake(s) impaired? If so, can the impairment(s) be linked to anthropogenic causes?

- Presence of forested areas, natural prairies or unaltered wetlands