| DATE : | March 22, 2006 |
| --- | --- |
| TO : | Effluent Limit Review file, City of Marshall Permit file, City of Marshall |
| thru | Greg Gross, Supervisor Water Standards Unit Marvin Hora, Manager Water Assessment and Environmental Information Section |
| FROM : | Mark Tomasek Water Standards Unit Environmental Analysis and Outcomes Division |
| PHONE : | 651/296-7241 |
| SUBJECT : | City of Marshall Phosphorus Effluent Limit Recommendation - updated |

The purpose of this memorandum is to provide a correction to an error in the March 22, 2006 memorandum from me to the same listed files as above.  The correction is to the determination of Lake Redwood volume.

Initial calculation of Lake Redwood volume was 207 acre-feet.  This calculation was in error due to misplaced parenthesis in the spreadsheet formula for the volume of specific depth contours.  The corrected formula yields a lake volume result of 125 acre-feet.  Recalculation of the mean hydraulic residence time at the 122Q10 (7.1 cfs) is 8.9 days.  This retention time is less than the 14 day criteria for lake determination.  This changes the consideration from the previous memorandum and determination of affects of the discharge from the City of Marshall on Lake Redwood is unwarranted.

Therefore, based on consideration of the city of Marshall discharge on Lake Redwood a phosphorus effluent limitation beyond that already in the Minnesota River Basin General Phosphorus Permit Phase I is not recommended.

It should be recognized that a bill passed in the 2006 Minnesota Legislative Session and signed by the Governor includes $1.6 million for the dredging of Lake Redwood.  Such dredging is anticipated to be initiated in 2007 and take 3 years to complete.  Following the dredging of Lake Redwood, lake volume and mean hydraulic detention times should be evaluated in subsequent permit reissuance in consideration of affects of the city of Marshall discharge on the lake.